TO DO LIST WINTER 2023/2024

FOR PARK ROLLING STOCK AND STRUCTURERS & WORKSHOP

- ALL CARRIAGES TO BE SERVICED CHECK ALL BEARINGS AND COMPONENTS. GREASE ALL UNITS. CHECK FOR WOOD DAMAGE REPAIR AND REPAINT AS REQUIRED. CHECK ALL CHAINS AND PINS ARE SECURE REPLACE PULL RINGS IF REQUIRED.CHECK ALL WHEELS ON CARRIAGES FOR WEAR OR DAMAGE.

- CHECK ALL AIRLINE CONNECTIONS ON EVA, STRATHAVEN CASTLE, ABBERTON, ALL SIX CARRIAGES, FISH VAN & R.S. WATT. TEST SINGLE UNITS AND MULIPLE UNITS FOR AIR PRESSURE (15 MINUTES MINIMUM HOLD). PUT BATTERIES ON REFRESH CHARGE. BUY REPLACEMENT BATTERIES AS REQUIRED.

- EVA BLOWDOWN AND OIL UP FOR WINTER. GREASE PONY WHEELS, CHECK GLANDS FOR PACKING REPACK IF REQUIRED TIGHTEN IF REQUIRED, REROUTE INJECTOR PIPE LINE TO BOILER. EVAS TENDER REQUIRES COMPLETE REWIRE & NEW AIR BRAKE SYSTEM TO BE INSTALLED (COMPRESSOR AND AIRLINES

- MARGARET BLOWDOWN AND OIL UP FOR WINTER.  CHECK GLANDS AND CYLINDERS FOR PACKING TIGHTEN IF REQUIRED. FIRE DOOR REQUIRES ATTENTION STUD REQUIRES REPLACING TO ALLOW FIRE DOOR TO CLOSE PROPERLY.  CHECK OVER TENDER AND ADJUST BRAKES IF REQUIRED.

- BOTH STEAM LOCOMOTIVES TO BE STEAM TESTED AND WHERE APPLICABLE HYDAULIC TESTING OF BOILER. SEPARATE RECORD HELD FOR THIS IN THE NEW YEAR.

- PUT ON WINTER HEATING IN KIOSK. DRAIN ALL WATER FROM SYSTEM, LEAVE TAPS OPEN.

- CLEAN OUT KIOSK PUT AWAY ALL ELECTRICAL CABLES, CLEAR ENGINE SHED AND KITCHEN.  CLEAN BACK WORKSHOP. THROW OUT ANY RUBBISH, NO FOOD STUFFS TO BE LEFT IN KIOSK.

- ABBERTON FULL-SERVICE CHECK AND REPLACE FLUIDS AS REQUIRED, CHAIN TENSION ETC.  GREASE TENDER WHEEL UNITS ETC. TAKE LOCO TO WORKSHOP CHECK AND REPLACE WHEELS AS REQUIRED. CHAINS AND SPROCKETS REQUIRE TO BE REPLACED ON RUNNING GEAR OF LOCO. DEGREASE CHASSIS WHILE LOCOMOTIVE IS STRIPPED DOWN. LOCO TO GET A FRESH COAT OF PAINT

- STRATHAVEN CASTLE FULL-SERVICE CHECK AND REPLACE ALL FLUIDS. HYDAULIC OIL CHANGE. CHECK CHAIN TENSION ETC. GREASE TENDER WHEEL UNITS

- FIONA REQUIRES TO BE BLOWNDOWN. BOTH DRIVING WHEELS ON THE RIGHT-HAND SIDE (LOOKING FROM CAB END OF LOCO) REQUIRE TO BE RE-ATTACHED TO THEIR AXLES CHECK TIMING. CHECK OVER 5” ROLLING STOCK AND FOR RAISED & GROUND TRACK OIL AND GREASE AS REQUIRED. CHECK PINS AND CHAINS IN PLACE.

- BATTERY DIESEL’S  NEPTUNE, CLASS 67, TITAN AND R.S. WATT BRAKE CARRIAGE TO BE SERVICED, OIL UP FOR WINTER. CHECK OVER WIRING AND REPLACE ANY FRAYED OR DAMAGED WIRES.  .

- TURNTABLE CLEAR PIT OF LEAVES AND DEBRIS, COSTING TO REPLACE FACING BRICKS OF T/TABLE.  SOURCE MORE GRANITE FOR TRACK BED AND RAISED TRACK.

- KEEP CLEARING UNWANTED ITEMS FROM WORKSHOP. ON GOING

- PLANT REPALACEMENT TREES FOR OUR BEACH TUNNEL, CUT AND PLACE PROTECTIVE SLEEVES AROUND THE BASE FOR THESE NEW TREES AND WHERE EXISTING TREES REQUIRE REPLACEMENTS PROTECTORS.

- CAIRNHILL REQUIRES STEAM TEST AND CHECK OVER ALL STEAM FITTINGS INJECTOR AND BLOWER. RETURN TO THE PARK THEREAFTER.

WHAT CAN YOU DO TO HELP COMPLETE THIS LIST??