TO DO LIST WINTER 2025/2026

FOR PARK ROLLING STOCK AND STRUCTURERS & WORKSHOP

- NEW CARRIAGES, NEW REPLACEMENT BOGIE PARTS TO BE FITTED TO ALL CARRIAGES. CHECK ALL CHAINS AND PINS ARE SECURE FIT PULL RINGS TO EACH SPLIT PIN. FINAL TESTING IN MARCH BEFORE NEW SEASON COMMENCES. WAXOIL TO BE RE-APPLIED TO ALL CARRIAGES IF REQUIRED. THREE OF THE OLD CARRIAGES HAVE BEEN KEPT AND SERVICING OF THEM MUST BE DONE BEFORE USE.THESE ARE BEING STORED AT WORKSHOP YARD.

- CHECK ALL AIRLINE CONNECTIONS ON EVA, STRATHAVEN CASTLE, ABBERTON, FISH VAN, R.S. WATT, AND NEW BRAKE CARRIAGE. TEST SINGLE UNITS AND MULIPLE UNITS FOR AIR PRESSURE (15 MINUTES MINIMUM HOLD). PUT BATTERIES ON REFRESH CHARGE. BUY REPLACEMENT BATTERIES AS REQUIRED.

- EVA BLOWDOWN AND OIL UP FOR WINTER. GREASE PONY WHEELS, CHECK GLANDS FOR PACKING REPACK IF REQUIRED TIGHTEN IF REQUIRED, STEAM LUBRICATOR REQUIRES CLEANING AND SERVICING. BOTH CLACK VALVES REQUIRE INSPECTION.

- MARGARET BLOWDOWN AND OIL UP FOR WINTER.  CHECK GLANDS AND CYLINDERS FOR PACKING TIGHTEN IF REQUIRED..  CHECK OVER TENDER AND ADJUST BRAKES IF REQUIRED. FIRE BOX DOOR REQUIRES ATTENTION. DRAIN COCKS LEVER REQUIRES RECONNECTION TO ROD TO ACTIVATE THE DRAIN COCKS.

- BOTH STEAM LOCOMOTIVES TO BE STEAM TESTED IN THE SPRING AND WHERE APPLICABLE HYDAULIC TESTING OF BOILER. SEPARATE RECORD HELD FOR THIS.

- PUT ON WINTER HEATING IN KIOSK. DRAIN ALL WATER FROM SYSTEM, LEAVE TAPS OPEN.

- CLEAN OUT KIOSK PUT AWAY ALL ELECTRICAL CABLES, CLEAR ENGINE SHED AND KITCHEN.  CLEAN BACK WORKSHOP. THROW OUT ANY RUBBISH, NO FOOD STUFFS TO BE LEFT IN KIOSK.

- ABBERTON FULL-SERVICE CHECK AND REPLACE FLUIDS AS REQUIRED, CHAIN TENSION AND CHECK RUBBER DRIVE BELTS FOR CRACKING ETC.  GREASE TENDER WHEEL UNITS ETC.

- STRATHAVEN CASTLE FULL-SERVICE CHECK AND REPLACE ALL FLUIDS. HYDAULIC OIL CHANGE. CHECK CHAIN TENSION ETC. GREASE TENDER WHEEL UNITS.

- FIONA REQUIRES TO BE BLOWNDOWN. DRIVING WHEELS NEED TO BE RE-ATTACHED TO THEIR AXLES CHECK TIMING ETC. PISTONS REQUIRE NEW PISTON RINGS

- MALDWYN AND CAIRNHILL BLOWDOWN AND OIL UP FOR WINTER.  CHECK GLANDS AND CYLINDERS FOR PACKING TIGHTEN IF REQUIRED. BOTH LOCO’S STEAM TEST AND CHECK OVER ALL STEAM FITTINGS INJECTOR ETC. TO GO TO THE PARK THEREAFTER.

- CAIRNHILL REQUIRES SPRINGS TO BE ADJUSTED FOR RAISED TRACK RUNNING.

- NEW 5” CARRAIAGES REQUIRE BOGIES AND UNDER CARRIAGES CLEANED. WAXOIL TO BE APPLIED

- BATTERY DIESEL’S NEPTUNE, CLASS 67, TITAN OIL UP WHERE APPLICACABLE. NEPTUNE REQUIRES NEW GEAR WHEEL TO BE FITTED TO REAR BOGIE.  CHECK OVER WIRING AND REPLACE ANY FRAYED OR DAMAGED WIRES. PUT BATTERIES ON REFRESH CHARGE.  METAL GUAZE FITTED OVER BOGIE TO PTOTECT PLASTIC GEARING

- TURNTABLE CLEAR PIT OF LEAVES AND DEBRIS.TRACK WORK AS IDENTIFIED ON GOING , TURNTABLE REQUIRES NEW TUBULAR SUPPORTS FOR TURNTABLE TRACK.

- KEEP CLEARING UNWANTED ITEMS FROM WORKSHOP. ON GOING

- CUT AND REPLACE PROTECTIVE SLEEVES AROUND THE BASE OF EXISTING BEECH TREES. TRIM WHERE APPLICABLE

- WORK ON BLACK 5 AND HOLMSIDE TO CONTINUE IN THE WORKSHOP

- I AM SURE WE FIND MORE ITEMS TO BE DONE BUT THIS GIVES US A GOOD STARTING POINT.

WHAT CAN YOU DO TO HELP COMPLETE THIS LIST??